Case Study: Interpolating the geoid height/undulation of a ground location or a space object from the world map of the geoid heights relate to World Geodetic System 1984 north/south latitudes and East/west longitudes

Prepared/Compiled/Edited by: XYZ

References:

- Wikipedia.com

Problem

Interpolating a geoid height NXY of a known point associated with the latitudes (X or ϕ) and longitude (Y or λ) from the given the World Geodetic System 1984 (WGS 84) geoid height in meters, the  NXY=fX,Y is a discrete function of X and Y in degrees and measured from the WGS 84 geoid or mean sea level surface to the ellipsoid surface (Fig 2)

# Data Sources

Latitude (X or ϕ)

Fig. 1 Longitude  (Y or λ)

NXY

NXY (Meters)

Table 1 contains entries of the World Geodetic System 1984 (WGS 84) geoid heights as a lookup table based on the geodetic latitudes and longitudes

# Table 1.  WGS- 84 Geoid Heights versus Geodetic Latitudes from +900 to -900 wrt Longitudes from -1800 to 1700

13,13,13,13,13,13,13,13,13,13,13,13,13,13,13,13,13,13,13,13,13,13,13,13,13,13,13,13,13,13,13,13,13,13,13,13

3,1,-2,-3,-3,-3,-1,3,1,5,9,11,19,27,31,34,33,34,33,34,28,23,17,13,9,4,4,1,-2,-2,0,2,3,2,1,1

2,2,1,-1,-3,-7,-14,-24,-27,-25,-19,3,24,37,47,60,61,58,51,43,29,20,12,5,-2,-10,-14,-12,-10,-14,-12,-6,-2,3,6,4

2,9,17,10,13,1,-14,-30,-39,-46,-42,-21,6,29,49,65,60,57,47,41,21,18,14,7,-3,-22,-29,-32,-32,-26,-15,-2,13,17,19,6

-8,8,8,1,-11,-19,-16,-18,-22,-35,-40,-26,-12,24,45,63,62,59,47,48,42,28,12,-10,-19,-33,-43,-42,-43,-29,-2,17,23,22,6,2

-12,-10,-13,-20,-31,-34,-21,-16,-26,-34,-33,-35,-26,2,33,59,52,51,52,48,35,40,33,-9,-28,-39,-48,-59,-50,-28,3,23,37,18,-1,-11

-7,-5,-8,-15,-28,-40,-42,-29,-22,-26,-32,-51,-40,-17,17,31,34,44,36,28,29,17,12,-20,-15,-40,-33,-34,-34,-28,7,29,43,20,4,-6

5,10,7,-7,-23,-39,-47,-34,-9,-10,-20,-45,-48,-32,-9,17,25,31,31,26,15,6,1,-29,-44,-61,-67,-59,-36,-11,21,39,49,39,22,10

13,12,11,2,-11,-28,-38,-29,-10,3,1,-11,-41,-42,-16,3,17,33,22,23,2,-3,-7,-36,-59,-90,-95,-63,-24,12,53,60,58,46,36,26

22,16,17,13,1,-12,-23,-20,-14,-3,14,10,-15,-27,-18,3,12,20,18,12,-13,-9,-28,-49,-62,-89,-102,-63,-9,33,58,73,74,63,50,32

36,22,11,6,-1,-8,-10,-8,-11,-9,1,32,4,-18,-13,-9,4,14,12,13,-2,-14,-25,-32,-38,-60,-75,-63,-26,0,35,52,68,76,64,52

51,27,10,0,-9,-11,-5,-2,-3,-1,9,35,20,-5,-6,-5,0,13,17,23,21,8,-9,-10,-11,-20,-40,-47,-45,-25,5,23,45,58,57,63

46,22,5,-2,-8,-13,-10,-7,-4,1,9,32,16,4,-8,4,12,15,22,27,34,29,14,15,15,7,-9,-25,-37,-39,-23,-14,15,33,34,45

21,6,1,-7,-12,-12,-12,-10,-7,-1,8,23,15,-2,-6,6,21,24,18,26,31,33,39,41,30,24,13,-2,-20,-32,-33,-27,-14,-2,5,20

-15,-18,-18,-16,-17,-15,-10,-10,-8,-2,6,14,13,3,3,10,20,27,25,26,34,39,45,45,38,39,28,13,-1,-15,-22,-22,-18,-15,-14,-10

-45,-43,-37,-32,-30,-26,-23,-22,-16,-10,-2,10,20,20,21,24,22,17,16,19,25,30,35,35,33,30,27,10,-2,-14,-23,-30,-33,-29,-35,-43

-61,-60,-61,-55,-49,-44,-38,-31,-25,-16,-6,1,4,5,4,2,6,12,16,16,17,21,20,26,26,22,16,10,-1,-16,-29,-36,-46,-55,-54,-59

-53,-54,-55,-52,-48,-42,-38,-38,-29,-26,-26,-24,-23,-21,-19,-16,-12,-8,-4,-1,1,4,4,6,5,4,2,-6,-15,-24,-33,-40,-48,-50,-53,-52

-30,-30,-30,-30,-30,-30,-30,-30,-30,-30,-30,-30,-30,-30,-30,-30,-30,-30,-30,-30,-30,-30,-30,-30,-30,-30,-30,-30,-30,-30,-30,-30,-30,-30,-30,-30

With the Python program shown in Table 2, the above data are read into a 19x37 array matrix. The entries are the values of the geoid heights:

- Column numbers  (C)  from 0 to 36 starting -1800 to +1700 increment by 100

- Row numbers (R) from 0 to 18 starting +900 to -900 decrement by 100

Note: the entries of longitude +1800 is equal to longitude -1800 (Earth longitude wraps around). For convenience, column 37 for the longitude +1800 may be added. Then the matrix becomes a 19x38 array matrix.

# Table 2. Example of Table 1 format wrt Python Programming

|  |  |  | LONGITUDE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | C | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 | 35 | 36 |
|  |  |  | d | -180 | -170 | -160 | -150 | -140 | -130 | -120 | -110 | -100 | -90 | -80 | -70 | -60 | -50 | -40 | -30 | -20 | -10 | 0 | 10 | 20 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | 110 | 120 | 130 | 140 | 150 | 160 | 170 | 180 |
| L A T I T U DE | R | d |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 0 | 90 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | 80 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 2 | 70 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 3 | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 4 | 50 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 5 | 40 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 6 | 30 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 7 | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 8 | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 9 | 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 10 | -10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 11 | -20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 12 | -30 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 13 | -40 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 14 | -50 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 15 | -60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 16 | -70 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 17 | -80 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 18 | -90 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

# Vertical Datum (Height) Parameter Definitions

The geoid height NXY  is a theoretical estimate based on the mathematical Earth model WGS 1984 to be defined as:

NXY=fX,Y=H-h

h=Geodetic or ellipsoid altitude (or height) from the WGS 84 ellipsoid model surface

H= Elevation or mean-sea level (MSL) altitude (or height) from the WGS 84 geoid model surface which approximates to the MSL surface. GPS cam provide the MSL altitude H

Because the vertical altitude (vertical datum) of a static of dynamic target is very important information for a static or dynamic aircraft when takeoff off , landing or low-altitude fly to avoid accidents, the relative altitude(either h or H) must be known. If a sensor such as the GPS provides a the MSL altitude H, then the geodetic altitude can be derived from

h=H-NXY or H=h+NXY

Physical Earth Surface

Fig 2 Relationships among ellipsoid, elevation (MSL), and geoid heights

- The latitude X is in the range values from +900 to -900with -100 decrement such as +900, +800, +700, ⋯,+100,+200,00,-100,-200,⋯,-700,-800,-900

  - X=00 which is called the equator (equatorial plane) divides the Earth to the northern and southern hemispheres

  - X=900 “North Pole” , rotation axis of the Earth  with counter clock wise (CCW) direction

  - X=-900 “South  Pole”

  - Positive X: 900≥X≥900, it’s called the northern latitudes denoted as “N” ; for example +47.1234560  , N47.1234560 or  N47007'24.442"

  - Negative X : 00≥X≥-900, it’s called the southern  latitudes denoted as “S” ; for example -47.1234560  , S47.1234560 or  S47007'24.442"

- Similarly, with respect to the order of the X range, the longitude Y is in the range values from -1800 to +1700with +100 increment such as -1800, -1700,-1600, ⋯,2100,-100,00,100,200,⋯,1500,1600,1700

  - Y=00 which is called the “prime” or “Greenwich” meridian or longitude ) divides the Earth in to the  eastern (right)  and western (left)  meridians/longitudes

  - Positive Y : +179.9990≥Y≥00, it’s called the eastern longitude denoted as “E” ; for example +47.1234560  , E47.1234560 or  E47007'24.442"

  - Negative Y: 00≥Y≥-1800, it’s called the southern  latitudes denoted as “S” ; for example -117.1234560  , W117.1234560 or  W117007'24.442"

  - Note:, Same  geoid heights  at  Y=±-1800

- If X / Y  values are given in radians or degrees: minutes:seconds formats, they must be converted to the decimal degrees for the interpolations.

# Algorithms

Refer to Wikipedia website for the bilinear interpolation for details and appendix A for the python program.

Interpolate a geoid height of an arbitrary location at the latitude and longitude (X,Y) with an assumption that (X,Y) is located within the rectangular with four corner indexes and their  geoid heights have been determined :N11=fX1,Y1, N12=fX1,Y2,N21=fX2,Y1,N22=fX2,Y2.

The bilinear-interpolation method is used [1]. In general, the four cases are considered for the interpolations

- Case 1 : X1<X<X2 and  Y1<Y<Y2  : Both X and Y directions require linear interpolations

- Case 2 : X=X1=X2 and  Y1<Y<Y2: Only Y direction requires  linear interpolation

- Case 3 : X1<X<X2 and  Y=Y1=Y2 : Only X direction requires  linear interpolation

- Case 4 : X=X1=X2  and  Y=Y1=Y2 : No interpolations require on both X and Y directions

# Testing Points

Case 1:

- (N30.123456, W175.123456)= (+30.123456, -175.123456)

- (N30.123456, E175.123456) = (+30.123456, +175.123456)

- (S30.123456, W175.123456) = (-30.123456, -175.123456)

- (S30.123456, E175.123456) = (-30.123456, +175.123456)

Case 2

- (N30, W175.123456)= (+30, -175.123456)

Case 3

- N30.123456, W170)= (+30.123456, -170)

Case 4

- N30, W170)= (+30, -170)

Case 2

Case 4

N

N

S

E

W

Case 1

Case 3

# Case 1: X1<X<X2 and  Y1<Y<Y2

The case 1 is a general case wrt the four red circle dots Xi,Yj  associate with their geoid heights Nij (i ,j=1,2)have been determined priory shown in Fig 3.

Latitude (North, N)

South (S)

West (W)

Longitude (East, E)

R2

R1

P, Case 1

,

N12

,

N11

,

Y2

Y :Case 1 Normal

Y1

fX2,Y1=N21

,

fX2,Y2=N22

,

X2

X1

X

Fig 3. Cases 1  X1<X<X2 and  Y1<Y<Y2   Interpolations

fX,Y1≈X2-XX2-X1fX1,Y1+X-X1X2-X1fX2,Y1⟹Coding:        N_X1=X2-X∙N11+X-X1∙N21X2-X1    (1)

fX,Y2≈X2-XX2-X1fX1,Y2+X-X1X2-X1fX2,Y2⟹Coding:         N_X2=X2-X∙N12+X-X1∙N21X2-X1      2

NXY=fX,Y≈Y2-YY2-Y1fX,Y1+Y-Y1Y2-Y1fX,Y2⟹Coding:         NXY=Y2-YY2-Y1NX1+Y-Y1Y2-Y1NX2        3

# Case 2: X=X1=X2 and  Y1<Y<Y2:

The case 2 contains the two red circle dots X,Yj  associate with their geoid heights NXj(j=1,2) have been determined shown in Fig 4. The interpolation requires on Y -axis only wrt given X. From the above equation 3,

NXY=fX,Y≈Y2-YY2-Y1fX,Y1+Y-Y1Y2-Y1fX,Y2⟹Coding:         NXY=Y2-YY2-Y1NX1+Y-Y1Y2-Y1NX2        3

Where NX1=fX,Y1 and NX2=fX,Y2  from Table 1 with the given X.

P, Case 2

,

X=X1=X2

Latitude

(North, N)

South (S)

West (W)

Longitude (East, E)

Y2

Y :Case 2

Y1

fX,Y1=NX1

,

fX,Y2=NX2

,

Fig 4. Cases 2:   X1=X=X2 and  Y1<Y<Y2   Interpolations

# Case 3: X1<X<X2 and  Y=Y1=Y2 :

The case 3 contains the two red circle dots XI,Y  associate with their geoid heights NiY (i =1,2) have been determined shown in Fig 5. The interpolation requires on Y -axis only wrt given X. From the above equation 3,

NXY=fX,Y≈Y2-YY2-Y1fX,Y1+Y-Y1Y2-Y1fX,Y2⟹Coding:         NXY=Y2-YY2-Y1NX1+Y-Y1Y2-Y1NX2        3

Where NX1=fX,Y1 and NX2=fX,Y2  from Table 1 with the given X.

fX1Y,=N1Y

,

P, Case 3

,

X2

Latitude

(North, N)

South (S)

West (W)

Longitude (East, E)

Y= Y1=Y2

fX2Y,=N2Y

,

Fig 5. Cases3:   X1<X<X2 and  Y1=Y= Y2  Interpolations

X1

X

NXY=fX,Y≈X2-XX2-X1fX1,Y+X-X1X2-X1fX2,Y⟹Coding:         NXY=X2-XX2-X1N1Y+X-X1X2-X1N2Y        4

# Case 4: No interpolations are needed

Provide NXY=fX,Y

# Appendix A: Python Programming Reference

https://gis.stackexchange.com/questions/15801/interpolate-undulations-for-given-points-using-a-geoid-model

import csv,sqlite3

'''this script inserts data stored in csv file into sqlite database'''

coordinates= csv.reader(open("geoid.csv"))

database = sqlite3.connect("geoid.db")

cursor = database.cursor()

database.execute("create table coords(name, lat INTEGER, lon INTEGER, und INTEGER);")

database.executemany("insert into coords(name,lat,lon,und) values (?, ?, ?, ?)",coordinates)

database.commit()

database.close()

Here's example input csv file, that contains ID,Latitude,Longitude,Undulation:
000001,54.9666669,18.7,28.442
000002,54.9166669,18.6833331,28.512
000003,54.9,18.6833331,28.523
000004,54.8833331,18.6666669,28.572
000005,54.8666669,18.6666669,28.585
000006,54.85,18.6666669,28.6
000007,54.8333331,18.0666669,29.891
000008,54.8333331,18.0833331,29.855
000009,54.8333331,18.1833331,29.637
000010,54.8333331,18.25,29.488
000011,54.8333331,18.2666669,29.45
000012,54.8333331,18.2833331,29.413
000013,54.8333331,18.3,29.376
This is numeric geoid model stored in ASCII csv file.

Now I would like to find four adjacent points for our example point:

import sqlite3

#find four adjacent points

def find_adjacent_coords(db, lat, lon, step=0.041667):

coords_range = lat-step, lat+step, lon-step, lon+step

return db.execute("""select lat, lon, und from coords where

lat > ? and lat < ? and

lon > ? and lon < ?""", coords_range).fetchall()

#connect database with gps points

database = sqlite3.connect('F.tsj')

cursor = database.cursor()

cursor.execute("select C1, C2, C3 from tblSoPoints")

#C1 is latitude

#C2 is longitude

#C3 is ellipsoid height

results = cursor.fetchall()

for line in results:

B = line[0]

L = line[1]

ellps_height = line[2]

#connect to geoid database

db = sqlite3.connect('geoid.db')

n = (find_adjacent_coords(db, a, b))

Example gps point:

54.4786674627, 17.0470721369, 86.5003132338

and adjacent points from geoid:

[(54.5, 17.041667, 31.993), (54.5, 17.083333, 31.911), (54.458333, 17.041667, 31.945), (54.458333, 17.083333, 31.866)]<br>

Then I would like to interpolate undulation for our point using these four adjacent points with known undulations to calculate orthometric height.

## Edit

Solution No 1

from __future__ import division

def bilinear_interpolation(x, y, points):

points = sorted(points)               # order points by x, then by y

(x1, y1, q11), (_x1, y2, q12), (x2, _y1, q21), (_x2, _y2, q22) = points

if x1 != _x1 or x2 != _x2 or y1 != _y1 or y2 != _y2:

raise ValueError('points do not form a square')

if not x1 <= x <= x2 or not y1 <= y <= y2:

raise ValueError('(x, y) not within the square')

return (q11 * (x2 - x) * (y2 - y) +

q21 * (x - x1) * (y2 - y) +

q12 * (x2 - x) * (y - y1) +

q22 * (x - x1) * (y - y1)

) / (x2 - x1) * (y2 - y1)

n = [(54.5, 17.041667, 31.993),

(54.5, 17.083333, 31.911),

(54.458333, 17.041667, 31.945),

(54.458333, 17.083333, 31.866),

]

B = 54.4786674627

L = 17.0470721369

print bilinear_interpolation(B, L, n)

import doctest

print doctest.testmod()

...and here's the solution No 2:

from __future__ import division

import doctest

def bilinear_interpolation(x, y, points):

# http://en.wikipedia.org/wiki/Bilinear_interpolation

points.sort()               # order by x, then by y

xs, ys, zs = zip(*points)

# verify it is a square

assert xs[0] == xs[1] and xs[2] == xs[3]

assert ys[0] == ys[2] and ys[1] == ys[3]

x1 = xs[0]

x2 = xs[2]

y1 = ys[0]

y2 = ys[1]

# verify interpolation versus extrapolation

assert x1 <= x <= x2, (x1, x, x2)

assert y1 <= y <= y2

q11, q12, q21, q22 = zs

return (q11 / ((x2 - x1) * (y2 - y1)) * (x2 - x) * (y2 - y) +

q21 / ((x2 - x1) * (y2 - y1)) * (x - x1) * (y2 - y) +

q12 / ((x2 - x1) * (y2 - y1)) * (x2 - x) * (y - y1) +

q22 / ((x2 - x1) * (y2 - y1)) * (x - x1) * (y - y1)

)

B = 54.5

L = 17.061667

n = [(54.5, 17.041667, 31.993),

(54.5, 17.083333, 31.911),

(54.458333, 17.041667, 31.945),

(54.458333, 17.083333, 31.866),

]

print doctest.testmod()

print bilinear_interpolation(B, L, n)

You have two tasks: (a) find the grid data for the vertices of a grid cell in which a GPS location (x,y) lies and (b) interpolate those data.

An efficient way to accomplish (a) is to store the grid as an array with a single index. Often grids are stored by rows with data progressing left to right within each row. A grid is determined by the coordinates of its origin, (x0,y0) (which will be (-180, -90) for a worldwide grid in decimal degrees), its cellsize c (which is 1/24 in your case), and the number of points per row (n). The values would be recorded in the sequence

(x0,y0),   (x0+c,y0),   (x0+2c,y0), ...,   (x0+(n-1)*c,y0)

(x0,y0+c), (x0+c,y0+c), (x0+2c,y0+c), ..., (x0+(n-1)*c,y0+c)

...

(x0,y0+(m-1)*c), (x0+c, y0+(m-1)*c), ..., (x0+(n-1)*c,y0+(m-1)*c)

This sequence can be addressed with a single index k = 0, 1, ..., n*m-1.

Given an arbitrary point (x,y), you use simple calculations to find the indexes where its four nearest neighbors are stored: they lie between rows i and i+1 and columns j and j+1 where

i = Floor((y-y0)/c), j = Floor((x-x0)/c).

The corresponding offsets are ni + j, n(i+1) + j, ni + (j+1), and n(i+1) + (j+1).

This assumes x0 <= x < x+nc and y0 <= y < y+mc, which is worth checking in a general-purpose program. The coordinates of these four points are readily seen to be equal to

(x0 + j*c, y0 + i*c), (x0 + j*c, y0 + (i+1*c), etc.

Now that you can solve (a) efficiently, it remains to interpolate the values found, which I will denote z00, z01, z10, and z11, situated as suggested in this crude picture (and shown more precisely in the figure at the end):

z01    z11

z

z00    z10

You wish to estimate the value of z. It is convenient to use bilinear interpolation. The formulas amount to the following: first interpolate linearly along the top and bottom edges of the square. Here, only the x coordinate is changing (from x0+j*c to x0+(j+1)*c). Rescale this to t varying from 0 to 1, where

t = (x - (x0+j*c))/c = (x-x0)/c - j.

The interpolated values are

z0 = (1-t)*z00 + t*z10, z1 = (1-t)*z01 + t*z11

Now interpolate vertically between z0 and z1. First rescale y to s varying from 0 to 1, where

s = (y-y0)/c - i

and find the interpolated value

z = (1-s)*z0 + s*z1.

### Example

Let (x0,y0) = (-180,-90), c = 1/24 = 0.041667, n = 24*360 = 8640, and (x,y) = (54.4786674627, 17.0470721369) (that is, near latitude 17 degrees north and longitude 54.5 degrees east). Then

i = Floor((y-y0)/c) = Floor(24*(90 + 17.047...)) = 2569

j = Floor((x-x0)/c) = Floor(24*(180 + 54.478....)) = 5627.

The undulation for (i,j), namely z00, is found at offset 8640*i+j = 48,619,849. The undulation for (i+1,j), namely z01, is found at offset 8640*(i+1)+j = 48,628,489. The undulation for (i,j+1), namely z10, is found at offset 48,619,850, and the undulation for (i+1,j+1), namely z11, is found at offset 48,628,490. Suppose these have the values given; that is,

z00 = 31.945, z01 = 31.993, z10 = 31.866, and z11 = 31.911

To interpolate, compute

t = (x-x0)/c - j = 0.488019

s = (y-y0)/c - i = 0.129731

Therefore

z0 = 31.96842 and z1 = 31.88796

Finally

z = 31.958.

| shareimprove this answer | answered Oct 16 '11 at 21:06 whuber♦ 54.9k11147224 |
| --- | --- |

| 2. | thank you very much for your complete lecture and your time. – daikini Oct 17 '11 at 15:25 |
| --- | --- |
|  | how the coordinates of origin of my geoid grid determine accuracy of computation? – daikini Oct 17 '11 at 19:51 |
|  | @user Inaccuracy in the origin translates directly to the same inaccuracy in the coordinates (x,y) of the GPS point. How much of a difference that makes depends on how variable the surrounding values z00, ..., z11 are. But usually there is no effective inaccuracy in grid coordinates: the grid is established, measurements are taken at the grid nodes, and any inaccuracy due to imprecise spatial location in the field is just rolled up into the overall inaccuracy of the measurements themselves. – whuber♦ Oct 17 '11 at 20:32 |
| 2. | For the record, check also geographiclib.sourceforge.net/cgi-bin/GeoidEval – markusN Dec 30 '11 at 14:45 |

add a comment

# Appendix B : Simulation Results